# IP & Networking Cheat Sheet — Questions + Short Answers

## 🔹 1. IP Fundamentals

Q1. What is an IP address?
A unique logical address used to identify a device on a network.

Q2. Difference between IPv4 and IPv6?
IPv4 = 32-bit, ~4.3B addresses; IPv6 = 128-bit, extremely large address space.

Q3. What is a subnet mask?
A mask that separates network and host portions of an IP address.

Q4. What is CIDR notation?
A method of representing subnet masks using “/prefix” (e.g., /24).

Q5. What is a default gateway?
A router that forwards packets to external networks.

Q6. Public vs Private IP ranges?
Private:
10.0.0.0/8
172.16.0.0/12
192.168.0.0/16
Public: All routable IPs outside private ranges.

Q7. What is an APIPA address?
169.254.x.x auto-assigned when DHCP fails.

## 🔹 2. Networking Basics

Q8. What is a MAC address?
A 48-bit physical address burned into the NIC.

Q9. Difference between Hub, Switch, Router?
Hub: Broadcasts
Switch: Forwards based on MAC
Router: Forwards based on IP

Q10. OSI vs TCP/IP?
OSI = 7 layers; TCP/IP = 4 layers (Application, Transport, Internet, Network Access)

Q11. What is MTU?
Maximum Transmission Unit — largest packet size allowed (default 1500 bytes).

Q12. What is ARP used for?
Mapping IP → MAC.

Q13. What is DNS?
Domain Name System — converts names to IPs.

## 🔹 3. Routing

Q14. What is routing?
Process of selecting the best path for packets across networks.

Q15. Static vs Dynamic Routing?
Static: Manually configured
Dynamic: Learns routes automatically via protocols

Q16. What are distance-vector protocols?
Routing based on hop-count; examples: RIP.

Q17. What are link-state protocols?
Routing based on full topology awareness; examples: OSPF, IS-IS.

Q18. What is BGP used for?
Inter-domain routing between ISPs; path-vector protocol.

Q19. What is an Autonomous System (AS)?
A network or group of networks managed under one administrative domain.

Q20. What is ECMP?
Equal Cost Multi-Path — load balancing over multiple equal-cost routes.

## 🔹 4. Switching & VLANs

Q21. What is a VLAN?
A virtual LAN created to segment networks at Layer 2.

Q22. What is the default VLAN?
VLAN 1 on most switches.

Q23. Access port vs Trunk port?
Access: Carries single VLAN
Trunk: Carries multiple VLANs (tagged)

Q24. What is 802.1Q?
Standard for VLAN tagging.

Q25. What is Native VLAN?
Untagged VLAN on a trunk link.

Q26. What is VLAN Trunking Protocol (VTP)?
Cisco protocol for VLAN configuration propagation.

## 🔹 5. VXLAN

Q27. What is VXLAN?
A network virtualization overlay that extends Layer 2 over Layer 3 networks.

Q28. What is the VXLAN header size?
50 bytes overhead.

Q29. What is a VNI?
VXLAN Network Identifier — 24-bit, supports 16 million segments.

Q30. Why use VXLAN?
Scalability, multi-tenancy, data center overlays.

Q31. What is VTEP?
VXLAN Tunnel Endpoint — encapsulates/decapsulates VXLAN packets.

Q32. Control-plane options in VXLAN?
Multicast, EVPN (preferred), static mapping.

## 🔹 6. DHCP

Q33. What is DHCP?
Protocol that automatically assigns IPs.

Q34. What are the 4 DHCP messages?
DORA — Discover, Offer, Request, Acknowledge.

## 🔹 7. NAT

Q35. What is NAT?
Translates private IPs to public IPs.

Q36. SNAT vs DNAT?
SNAT: Source address translation
DNAT: Destination address translation

Q37. What is PAT?
Port Address Translation — many-to-one NAT.

## 🔹 8. Firewalls & ACLs

Q38. What is a firewall?
Security device enforcing rules on traffic flow.

Q39. What is an ACL?
Access Control List — allows/denies traffic.

## 🔹 9. IP Services

Q40. What is ICMP?
Protocol for errors & diagnostics (ping, traceroute).

Q41. What is QoS?
Traffic prioritization using marking, queuing, shaping.

## 🔹 10. Subnetting Quick Qs

Q42. How many hosts in /24?
254

Q43. How many hosts in /30?
2

Q44. What is the subnet mask for /26?
255.255.255.192

Q45. How many subnets in a /28?
16-address blocks (14 usable hosts each).

## 🔹 11. MPLS Basics

Q46. What is MPLS?
Label-based forwarding used in service provider networks.

Q47. What is an LSP?
Label Switched Path — pre-established forwarding path.

## 🔹 12. Ethernet Basics

Q48. What is full-duplex?
Send and receive simultaneously.

Q49. What is auto-negotiation?
Device capability exchange (speed/duplex).

## 🔹 13. Troubleshooting

Q50. What does traceroute show?
Hop-by-hop path of packets.

Q51. When do you use ping?
To test reachability and RTT.